TBB Apollo Maxx series advanced photovoltaic inverter control all-in-one machine (supporting parallel three-phase)

Advanced All in One Solar Inverter With parallel and three phase capability Apollo Maxx 2KW-5KW,Apollo Maxx is a powerful all in one solar inverter that integrates with multiple functions, including a high-performance true sine wave inverter, a powerful battery charger,a MPPTcharge controller,a high-speed automatic transfer switch and two outputs for load management.

Apollo Maxx 2.0M           Apollo Maxx3.0M

Apollo Maxx 2.0S           Apollo Maxx3.0S

Apollo Maxx5.0S

---------------------------------------------------------------------------------------------------------------------------------

Product parameters   Application   Featured Services  Certificate&Shipping

Features

Product Details

- Product parameters

| Model No. | Apollo Maxx2.0M | Apollo Maxx3.0M | Apollo Maxx2.0S | Apollo Maxx3.0S | Apollo Maxx5.0S |
| --- | --- | --- | --- | --- | --- |
| Product Topology | Transformer based |  |  |  |  |
| Power Assist | Yes |  |  |  |  |
| Parallel & Three Phase | Yes |  |  |  |  |
| AC input range | 175~265VAC(45~65Hz) |  |  |  |  |
| AC input Current (transfer switch)(A) | 32 |  |  |  | 50 |

| Nominal battery voltage (M) 24 |  |  | 48 |  |  |
| --- | --- | --- | --- | --- | --- |
| Input voltage range (V) 21~34 42~68 |  |  |  |  |  |
| AC output voltage (VAC) 220/230/240±2% |  |  |  |  |  |
| AC output Frequency (Hz) 50/60±0.1% |  |  |  |  |  |
| Harmonic distortion <2% |  |  |  |  |  |
| Load Powerfactor 1.0 |  |  |  |  |  |
| Cont.output power at 25℃(VA) | 2000 | 3000 | 2000 | 3000 | 5000 |
| Max output power at 25℃(M) | 2000 | 3000 | 2000 | 3000 | 5000 |
| Peak power(5 sec)(M) | 6000 | 9000 | 6000 | 9000 | 15000 |
| Surge | 300% |  |  |  |  |
| Maximum efficiency | 94% | 94% | 95% | 95% | 96% |
| Zero load power (M) | 11 | 14 | 11 | 14 | 21 |

| Max output current(A) | 60 |  | 60 | 60 | 60 |  | 90 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum PV power (M) | 2000 |  | 2000 | 4000 | 4000 |  | 6000 |  |
| PV open circuit voltage (M) | 150 |  |  |  |  |  |  |  |
| MPPTvoltage range (M | 65~145 |  |  |  |  |  |  |  |
| Max.Py short circuit current (A) | 18 | 18 |  | 35 |  | 35 |  | 54 |
| Charge voltage'absorption'(V) | 28.8 |  |  | 57.6 |  |  |  |  |
| Charge voltage 'float'(V) | 27.6 |  |  | 55.2 |  |  |  |  |
| MPPT charger maximum efficiency | 98% |  |  |  |  |  |  |  |
| MPPTefficiency | >99.5% |  |  |  |  |  |  |  |
| Protection | a) output short circuit; b) overload; c) battery voltage too high d) battery voltage too low; e) temperature too high; f) input voltage out of range |  |  |  |  |  |  |  |

- -------------------------------------------------------------------------------------------------------

- 2.

- ------------------------------------------------------------------------------------------------------------------------------

- 3.servive

- OEM/ODM

- Product label

- Longrun prides itself on helping customers enhance their

- private label product lines.Whether you need help creating

- the right formula or have a range of products you want to

- compete with, we can help you deliver high-quality products

- every time.

- Packing of contract

- Longrun can also be an extension of your company If

- you already have an amazing product but can’t package

- and ship it exactly as you want.We offer contract packaging

- that can easily fill gaps in areas of your business that you

- cannot currently complete

FAQS

1.Can I have my own custom design for products and packaging?

Yes, you can use OEM according to your needs. Just give us the artwork you designed

- 2.What’s the lead time for mass production？

- -- It depends on the actual situation. 48V100ah LFP battery pack,3-7 days with stock, if without stock, that will be depends on your order quantity, normally needs 20-25 days.

- 3.How's your quality control system?

- -- 100% PCM test by IQC.

- -- 100% Capacity test by OQC.

- 4.How's the lead time and services?

- -- Fast Delivery in 10 days.

- -- 8h response & 48h solution.

- .-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

- 4.

- At present, the company is vigorously expanding its overseas market and making a global layout. In the next three years, we are committed to becoming one of the top ten new energy battery export enterprises in China, serving the world with high-quality products, and achieving win-win results with more customers.

·Delivery within 48 hours

----------------------------------------------------------------

FAQS

1.Can I have my own custom design for products and packaging?

Yes, you can use OEM according to your needs. Just give us the artwork you designed

- 2.What’s the lead time for mass production？

- -- It depends on the actual situation. 48V100ah LFP battery pack,3-7 days with stock, if without stock, that will be depends on your order quantity, normally needs 20-25 days.

- 3.How's your quality control system?

- -- 100% PCM test by IQC.

- -- 100% Capacity test by OQC.

- 4.How's the lead time and services?

- -- Fast Delivery in 10 days.

- -- 8h response & 48h solution.